| EX NO: 2 | Configure VLANs and inter-VLAN routing in Packet Tracer |
| --- | --- |
| DATE:21/07/26 |  |

# Objectives:

- To create and configure Virtual Local Area Networks (VLANs) for logical network segmentation and traffic isolation.

- To configure Inter-VLAN Routing to enable communication between devices belonging to different VLANs.

# Software/Apparatus required:

  - Cisco Packet Tracer

  - End Devices (PCs)

  - Switches

  - Copper Straight-through Cable / Automatic Cable Selector

  - Routers

# Procedure

Objective 1: VLAN Configuration

    - Open Cisco Packet Tracer.

    - Place two switches and six PCs in the workspace.

    - Connect the PCs to the switches using straight-through cables.

    - Connect the two switches using a crossover cable.

    - Create VLAN 10 and VLAN 20 on both switches.

    - Assign the ports connected to PC0, PC1, and PC2 to VLAN 10.

    - Assign the ports connected to PC3, PC4, and PC5 to VLAN 20.

    - Configure the link between the switches as a trunk link.

    - Configure the PCs with the required network settings.

    - Verify communication between devices belonging to the same VLAN.

    - Verify that communication between devices in different VLANs is not possible.

    - Save the configuration and observe the results.

Objective 2: Inter-VLAN configuration

- Open Cisco Packet Tracer.

- Place one router, three switches, and six PCs in the workspace.

- Connect the PCs to their respective switches using straight-through cables.

- Connect the access switches to the central switch.

- Connect the router to the central switch.

- Create VLAN 10 and VLAN 20 on the switches.

- Assign the ports connected to PC0, PC1, and PC2 to VLAN 10.

- Assign the ports connected to PC3, PC4, and PC5 to VLAN 20.

- Configure the links between the switches as trunk links.

- Configure the router interfaces to route traffic between VLAN 10 and VLAN 20.

- Configure the required network settings on all PCs.

- Verify communication between devices within the same VLAN.

- Verify communication between devices belonging to different VLANs through the router.

- Save the configuration and observe the results. 15.

# Networking diagram:

Objective 1: VLAN Configuration

Objective 2: Inter-VLAN Configuration

# Output:

Objective 1: VLAN Configuration

| SI. No | Source IP | Destination IP | Data sent (bytes) | Data received (bytes) |
| --- | --- | --- | --- | --- |
| 1. | 192.168.10.1 | 192.168.10.2 | 4 | 4 |
| 2. | 192.168.10.2 | 192.168.10.3 | 4 | 4 |
| 3. | 192.168.10.3 | 192.168.10.4 | 4 | 4 |

Objective 2: Inter-VLAN Configuration

| SI. No | Source IP | Destination IP | Data sent (bytes) | Data received (bytes) |
| --- | --- | --- | --- | --- |
| 1. | 192.168.10.1 | 192.168.20.1 | 4 | 4 |
| 2. | 192.168.10.2 | 192.168.20.2 | 4 | 4 |
| 3. | 192.168.10.3 | 192.168.20.3 | 4 | 4 |

# Result:

Objective 1: VLAN Configuration

The VLANs were successfully configured, and communication was established only between devices within the same VLAN while communication between different VLANs was restricted.

Objective 2: Inter-VLAN Configuration

- Inter-VLAN Routing was successfully configured, enabling communication between devices belonging to different VLANs through the router.

# Test Case Question 1

Implement the specified VLAN topology using Cisco Packet Tracer and analyze the communication between devices.

# Case 1

  - VLANs: VLAN 10 and VLAN 20

  - Number of PCs: 4 (2 PCs per VLAN)

  - Number of Switches: 1

# Case 2

  - VLANs: VLAN 10 and VLAN 20

  - Number of PCs: 6 (3 PCs per VLAN)

  - Number of Switches: 2

  - Inter-switch Connection: Trunk Link

# Tasks

- Construct the specified VLAN topology and configure the required VLANs and switch port assignments.

- Configure the network settings of all end devices and verify communication within the same VLAN.

- Analyze the communication behavior between devices belonging to the same VLAN and different VLANs.

| Aim (10) | Networking Diagram (30) | Procedure (10) | Results (20) | Record (10) | Viva (20) | Total (100) |
| --- | --- | --- | --- | --- | --- | --- |

# Test Case Question 2

Implement the specified Inter-VLAN Routing topology using Cisco Packet Tracer and analyze communication between different VLANs.

# Case 1

  - VLANs: VLAN 10 and VLAN 20

  - Number of PCs: 4 (2 PCs per VLAN)

  - Number of Switches: 2

  - Number of Routers: 1

# Case 2

  - VLANs: VLAN 10 and VLAN 20

  - Number of PCs: 6 (3 PCs per VLAN)

  - Number of Switches: 3

  - Number of Routers: 1

# Tasks

- Construct the specified Inter-VLAN Routing topology and configure the required VLANs, trunk links, and router interfaces.

- Configure the network settings and default gateways of all end devices and verify communication between VLANs.

- Analyze the effectiveness of Inter-VLAN Routing in enabling communication between different VLANs and compare it with VLAN-only communication.

| Aim (10) | Networking Diagram (30) | Procedure (10) | Results (20) | Record (10) | Viva (20) | Total (100) |
| --- | --- | --- | --- | --- | --- | --- |