| EX NO: 5 | Implementation of an IoT-Based Smart Home Automation Network Using Cisco Packet Tracer |
| --- | --- |
| DATE:08/08/26 |  |

# Objectives

- To design and configure an IoT smart home network with PC-based monitoring and control, integrating IoT devices, an access point, a switch, and an IoT server to enable centralized management of smart appliances.

- To establish smartphone-based Wi-Fi connectivity for remote access and control of IoT devices, and verify wireless communication between the mobile device and the smart home network through the access point and IoT server.

# Software/Apparatus required:

- Cisco Packet Tracer

- End Devices (PCs)

- Switches

- Copper Straight-through Cable / Automatic Cable Selector

- Routers

- Server (DHCP)

- Household devices (Fan, AC, Light, Garage door)

# Procedure:

Objective 1: PC based monitoring

  - Open Cisco Packet Tracer and create a new workspace.

  - Add a Home Gateway/Access Point, Switch, Server, PC, and IoT devices (Fan, Air Conditioner, Garage Door, and Light).

  - Connect the Server and Home Gateway to the Switch using copper straight-through cables.

  - Configure IP addressing for the Server and PC.

  - Enable IoT services on the Server and create a user account.

  - Connect the IoT devices wirelessly to the Home Gateway.

  - Register all IoT devices with the IoT Server.

  - Verify that the devices appear in the IoT Server dashboard.

  - Access the IoT Server from the PC using a web browser.

  - Monitor and control the IoT devices through the PC interface and verify their operation. Objective 2: Smartphone-based monitoring

- Open the configured IoT smart home topology in Cisco Packet Tracer.

- Add a Smartphone and ensure the Home Gateway/Access Point is active.

- Configure the wireless network (SSID and password) on the Home Gateway.

- Connect the Smartphone to the Wi-Fi network using the configured SSID.

- Ensure the Smartphone obtains an IP address automatically.

- Open the IoT monitoring application or web browser on the Smartphone.

- Log in to the IoT Server using the registered user credentials.

- Access the list of connected IoT devices.

- Remotely monitor and control the Fan, Air Conditioner, Garage Door, and Light using the Smartphone.

- Verify that the device status changes according to the commands issued from the Smartphone.

Objective 1: PC based monitoring

Objective 2: Smart phone based monitoring

Objective 1: PC based monitoring

| Things | Default | Control |
| --- | --- | --- |
| Fan | Off | ON |
| AC | Off | ON |
| Garage Door | Open | CLOSED |
| Light | Dim | BRIGHT |

Objective 2: Smart phone based monitoring

| Things | Default | Control |
| --- | --- | --- |
| Fan | Off | ON |
| AC | Off | ON |
| Garage Door | Open | CLOSED |

| Light | Dim |  |
| --- | --- | --- |

# Result

Objective 1: PC based monitoring

The IoT smart home network was successfully configured, and the PC was able to monitor and control the connected IoT devices through the server.

Objective 2: Smart phone based monitoring

The smartphone was successfully connected to the Wi-Fi network and was able to remotely monitor and control the IoT devices.

# Test Case Question 1

Implement the specified IoT Smart Home topology using Cisco Packet Tracer and analyze PC-based monitoring and control of IoT devices through an IoT server.

Case 1

- Number of PCs: 1

- Number of Switches: 1

- Number of Home Gateways/Access Points: 1

- Number of IoT Servers: 1

- Number of IoT Devices: 4 (Fan, Air Conditioner, Garage Door, Light)

- Access Method: PC-Based Monitoring Case 2

- Number of PCs: 2

- Number of Switches: 1

- Number of Home Gateways/Access Points: 1

- Number of IoT Servers: 1

- Number of IoT Devices: 6 (Fan, Air Conditioner, Garage Door, Light, Webcam, Smoke Detector)

- Access Method: PC-Based Monitoring Tasks

- Construct the specified topology and configure the IoT server and home gateway.

- Register the IoT devices with the IoT server and establish network connectivity.

- Access the IoT server from the PC and verify monitoring and control of all IoT devices.

# Test Case Question 2

Implement the specified IoT Smart Home topology using Cisco Packet Tracer and analyze smartphone-based Wi-Fi monitoring and control of IoT devices.

Case 1

  - Number of Smartphones: 1

  - Number of Switches: 1

  - Number of Home Gateways/Access Points: 1

  - Number of IoT Servers: 1

  - Number of IoT Devices: 4 (Fan, Air Conditioner, Garage Door, Light)

  - Access Method: Mobile Wi-Fi Control Case 2

  - Number of Smartphones: 2

  - Number of Switches: 1

  - Number of Home Gateways/Access Points: 1

  - Number of IoT Servers: 1

  - Number of IoT Devices: 6 (Fan, Air Conditioner, Garage Door, Light, Webcam, Smoke Detector)

  - Access Method: Mobile Wi-Fi Control

  - Wireless Network: Secured Wi-Fi Connection Tasks

- Construct the specified topology and configure the IoT server, home gateway, and Wi-Fi settings.

- Connect the smartphones to the wireless network and register the IoT devices with the server.

- Access the IoT server from the smartphones and verify remote monitoring and control of all IoT devices